Strain Gauge WIM Terminal

Project Documentation

ADJ Engineering Pvt. Ltd.
Last updated: 2026-05-08

# 1. Project Overview

This project is a Weigh-In-Motion (WIM) terminal for a 4-sensor load cell scale used by ADJ Engineering Pvt. Ltd. It reads weight data from a physical weighing terminal (IND570 or compatible), converts it, and displays it in a GUI.

## Data Flow

Physical Scale (4 load cells)
        │
        │  USB Serial  (45-byte binary packets)
        ▼
  [read_usb.py]          ← low-level USB reader / debugger
  [basic_interface.c]    ← GTK GUI reading raw USB serial packets
        │
        │  TCP stream (plain-text kg)  192.168.0.1:1702
        ▼
  [reader.c]             ← terminal live viewer (CLI)
  [bridge.c]             ← converts TCP stream → serial binary packets
  [gui_interface.py]     ← main WIM Terminal GUI (Python/Tkinter)
        │
        │  Virtual serial port  /tmp/ttyV0 ↔ /tmp/ttyV1
        ▼
  Company software (TVEMA or equivalent)

# 2. Hardware Setup

| Component | Detail |
| --- | --- |
| Scale | 4-corner load cell platform |
| Sensors | 4 × Wheatstone bridge strain gauges (S0–S3) |
| Each sensor | 4 bridge arms (P1–P4) |
| Controller | IND570 weighing terminal |
| USB output | 45-byte binary packets @ 115200 baud |
| TCP output | Plain-text weight stream on port 1702 |
| Host machine IP | 192.168.0.100 |
| Terminal IP | 192.168.0.1 |

# 3. Packet Format

Every measurement is transmitted as a 45-byte binary packet over USB serial or virtual serial. All multi-byte values are little-endian signed int16.

Byte  0        : 0x80          ← start-of-frame header
Bytes  1 – 11  : Group 0       ← 0xC0 | cfg(2) | P1(2) | P2(2) | P3(2) | P4(2)
Bytes 12 – 22  : Group 1       ← 0xC1 | cfg(2) | P1(2) | P2(2) | P3(2) | P4(2)
Bytes 23 – 33  : Group 2       ← 0xC2 | cfg(2) | P1(2) | P2(2) | P3(2) | P4(2)
Bytes 34 – 44  : Group 3       ← 0xC3 | cfg(2) | P1(2) | P2(2) | P3(2) | P4(2)

cfg = 2-byte hardware ADC config register, constant per channel, not used in weight calculation.  P1–P4 = raw ADC counts from the 4 arms of the Wheatstone bridge for that sensor.

## Packet Validation

A packet is valid when all of the following are true:

byte[0]  == 0x80
byte[1]  == 0xC0
byte[12] == 0xC1
byte[23] == 0xC2
byte[34] == 0xC3

# 4. Calibration

Each sensor has 4 Wheatstone bridge arms. Weight is derived by comparing each arm's raw count against a known zero offset and sensitivity.

## Formula

arm_kg[a]  = (P[a] - ZERO[sensor][a]) / SENS[sensor][a]
sensor_kg  = mean(arm_kg[0..3])
total_kg   = sensor_kg[0] + sensor_kg[1] + sensor_kg[2] + sensor_kg[3]

## Zero Offsets (raw counts at 0 kg, captured 2026-04-15)

| Sensor | P1 | P2 | P3 | P4 |
| --- | --- | --- | --- | --- |
| S0 | -755.0 | -2594.7 | 1361.0 | -3485.7 |
| S1 | -752.3 | -2595.0 | 1359.3 | -3483.7 |
| S2 | -751.0 | -2595.7 | 1359.3 | -3483.0 |
| S3 | -749.0 | -2597.3 | 1357.0 | -3483.7 |

## Sensitivity (counts per kg, derived from 665 kg capture)

| Sensor | P1 | P2 | P3 | P4 |
| --- | --- | --- | --- | --- |
| S0 | 1.6186 | 1.2491 | 1.0482 | 0.9479 |
| S1 | 1.6026 | 1.2511 | 1.0585 | 0.9360 |
| S2 | 1.5947 | 1.2555 | 1.0586 | 0.9314 |
| S3 | 1.5825 | 1.2652 | 1.0723 | 0.9357 |

Note: Calibration is based on a single-point 665 kg capture. Accuracy is best above ~200 kg. Noise floor near zero is approximately ±5 kg.

# 5. File Reference

| File | Language | Purpose |
| --- | --- | --- |
| gui_interface.py | Python | Main WIM Terminal GUI (CH1 Tones + Result) |
| read_usb.py | Python | Low-level USB serial reader and packet debugger |
| basic_interface.c | C / GTK3 | 4-channel GTK GUI reading direct USB serial |
| reader.c | C | CLI live weight viewer connecting to TCP stream |
| bridge.c | C | Converts TCP weight stream to binary serial packets |
| replay.c | C | Replays captured binary frames to test serial |
| check_packet.c | C | Compares generated packets against captured frames |
| Makefile | Make | Builds all C tools |
| install_windows.bat | Batch | Windows installation helper |
| adj_logo_small.png | Image | Logo shown in GUI header |
| hex_dumps.txt | Text | Reference hex captures for packet analysis |
| StrainGauge.spec | PyInstaller | Spec file for building standalone executable |

# 6. Building the C Tools

## Prerequisites

# Ubuntu / Debian
sudo apt install build-essential libgtk-3-dev

# Fedora / RHEL
sudo dnf install gcc gtk3-devel

## Build All

make

## Clean

make clean

# 7. Running Each Tool

## read_usb.py — USB packet debugger

python3 read_usb.py                           # default /dev/ttyUSB0 @ 115200
python3 read_usb.py --port /dev/ttyUSB1
python3 read_usb.py --raw                     # also print raw hex bytes
python3 read_usb.py --baud 9600 19200 115200  # auto-probe baud rate

## reader — CLI live weight viewer

./reader

| Key | Action |
| --- | --- |
| 1 | Mode 1 — real-time kg |
| 2 | Mode 2 — 1 kg = 1 ton |
| 3 | Mode 3 — 1 kg = 2 ton |
| Ctrl+C | Stop |

## bridge — TCP to serial bridge

Converts the TCP plain-text weight stream to binary 45-byte packets for the company software.

# Step 1 — create virtual serial port pair
socat -d -d pty,raw,echo=0,link=/tmp/ttyV0 pty,raw,echo=0,link=/tmp/ttyV1

# Step 2 — connect company software to /tmp/ttyV1

# Step 3 — run the bridge
./bridge

## replay — serial frame replayer

Sends captured binary frames in a loop at 100 Hz to test the serial connection without a live scale.

./replay

## check_packet — packet format verification

./check_packet

## basic_interface — GTK 4-channel interface

./basic_interface

## gui_interface.py — Main WIM Terminal GUI

python3 gui_interface.py

# 8. WIM Terminal GUI (gui_interface.py)

## Layout

┌─────────────────────────────────────────────────────────┐
│  [Logo]  WIM Terminal          Connection ●  ✕ Close    │
├──────────────────────────┬──────────────────────────────┤
│  CH 1                    │  Result                      │
│  ┌──────────────────┐    │  S = ┌──────────┐  kg        │
│  │  Tones           │    │      │          │            │
│  └──────────────────┘    │  MODE [kg → kg (Δ) ▾]        │
│                          │  [Δ Delta]  [⊙ Zero]         │
├──────────────────────────┴──────────────────────────────┤
│  Average by [100 ↕] measurements       Bytes received:  │
└─────────────────────────────────────────────────────────┘

## Panels

CH 1 (left panel): Shows the Tones value for channel 1. Value changes based on the selected mode and Delta base.

Result (right panel): S= shows the live result value in the selected unit. MODE dropdown selects the conversion mode. Δ Delta button captures the current weight as a base. ⊙ Zero button zeroes both displays.

Bottom bar: Average window spinner (1–1000 measurements) and live byte count / error message.

## Connection Indicator

| Indicator | Meaning |
| --- | --- |
| Red ● | Not connected or error |
| Green ● | Receiving live data |

# 9. Network & Serial Configuration

## TCP

| Parameter | Value |
| --- | --- |
| Terminal IP | 192.168.0.1 |
| Port | 1702 |
| Host machine IP | 192.168.0.100 |
| Protocol | Plain ASCII, one weight value per line |
| Supported units | kg, g, lb, t (all converted to kg internally) |

## USB Serial

| Parameter | Value |
| --- | --- |
| Port | /dev/ttyUSB0 |
| Baud rate | 115200 |
| Data bits | 8 |
| Parity | None |
| Stop bits | 1 |

If permission is denied on the serial port:

sudo usermod -aG dialout $USER
# log out and back in

# 10. Modes Explained

| Mode | S = (Result) | CH1 Tones |
| --- | --- | --- |
| kg → kg (Δ) | Live kg | Live kg (or delta decimal if base set) |
| kg → Tons (Δ) | Live kg | Live kg (or delta decimal if base set) |
| kg → 2×kN (Δ) | Live kg × 2 | Live kg (or delta decimal if base set) |
| kg → 2kg (Δ) | Live kg × 2 | Live kg (or delta decimal if base set) |

# 11. Delta (Δ) Feature

The Delta feature captures a base weight at a point in time. After that, CH1 Tones displays the weight as a small decimal offset from that base, making it easy to see small changes on a large load.

## How to Use

1.  Wait for a stable reading.

2.  Press Δ Delta — the current weight is saved as the base.

3.  CH1 Tones now shows:  base + (current − base) / 1000

## Example

Base = 500 kg, current = 520 kg

CH1 Tones = 500 + (520 - 500) / 1000 = 500.020

A 20 kg change appears as 0.020 in the decimal portion — useful for monitoring small incremental loads on a large base. To reset the delta, press ⊙ Zero.

# 12. Zero Feature

Pressing ⊙ Zero does the following:

Captures the current live weight as a tare offset — all subsequent readings are relative to this point.

Clears the Delta base.

Both displays are immediately set to 0.000 / 0.00.

After 10 seconds, the displays resume showing live weight relative to the new tare.

The 10-second freeze gives time to remove whatever was on the scale before taring, or to confirm the zero visually.

# 13. Troubleshooting

## GUI shows "ERROR: Cannot connect"

Check that the IND570 terminal is powered on and connected to the network.

Verify the host machine IP is 192.168.0.100 and can reach 192.168.0.1.

Check firewall rules for port 1702.

ping 192.168.0.1
telnet 192.168.0.1 1702

## read_usb.py shows no packets

Check USB cable and port: ls /dev/ttyUSB*

Try --baud 9600 19200 115200 to auto-probe the baud rate.

Check permissions: sudo usermod -aG dialout $USER

## bridge cannot open serial port

Make sure socat is running first (creates /tmp/ttyV0 and /tmp/ttyV1).

Check SERIAL_PORT in bridge.c matches the socat output path.

## Weight reads incorrectly near zero

Calibration is accurate above ~200 kg; noise floor near zero is ±5 kg.

Use the ⊙ Zero button to tare after the scale has warmed up.

## CH1 Tones shows unexpected decimal values

A Delta base may be set — press ⊙ Zero to clear it.

Check the 'Base:' label below the Delta button to see if a base is active.